# Practical : 20

Aim: WAP to interface external LED with Breadboard & Arduino and write a program to Fade LED With Arduino Analog Output.

## Objectives:

- To learn Arduino UNO basics.

- To Learn Breadboard basics.

- Write a program to blink Arduino onboard LED and to interface external LED with Breadboard & Arduino.

- Write a program to Fade LED With Arduino Analog Output.

Components: Arduino UNO R3,Breadboard,1 Resistor,1 LED, Wires

## Circuit:

Program Code:

int brightness = 0; void setup()

{

pinMode(9, OUTPUT);

}

void loop()

{

for (brightness = 0; brightness <= 1000; brightness += 5)

{

analogWrite(9, brightness);

delay(50); // Wait for 30 millisecond(s)

}

for (brightness = 1000; brightness >= 0; brightness -= 5)

{

analogWrite(9, brightness);

delay(50); // Wait for 30 millisecond(s)

}

}

## Output:

Conclusion:

Thus, learnt about basic components of IoT like Arduino UNO (blinking Arduino onboard LED), Breadboard , LED’s and interfacing Fade LED With Arduino Analog Output with connections of resistors, breadboard, Jumper wires & Arduino.

# Practical : 21

Aim: WAP to interface Push Button with Arduino and write a program to turn ON LEDs when push button is pressed and gets OFF when push button is pressed again.

## Objectives:

- To learn Arduino UNO basics.

- To Learn Breadboard basics.

- To Learn Push buttons / switches.

- Programming of interfacing Push Button with Arduino

- Write a program to turn ON LEDs when push button is pressed and gets OFF gets OFF when push button is pressed again.

Components: Arduino UNO R3,Breadboard,4 Resistor,3 LED, 1 Push Button, Wires

## Circuit:

Program Code:

int ledPin1 = 10; int ledPin2 = 11; int ledPin3 = 12; int keyPin = 2;

bool ledState = false; // Track LED ON/OFF state

bool lastButtonState = HIGH; // Store previous button state void setup()

{

pinMode(ledPin1, OUTPUT); pinMode(ledPin2, OUTPUT); pinMode(ledPin3, OUTPUT);

pinMode(keyPin, INPUT_PULLUP); // Enable internal pull-up resistor

}

void loop()

{

bool currentButtonState = digitalRead(keyPin);

// Detect button press (falling edge: HIGH to LOW)

if (lastButtonState == HIGH && currentButtonState == LOW)

{

ledState = !ledState; // Toggle LED state digitalWrite(ledPin1, ledState ? HIGH : LOW); delay(500);

digitalWrite(ledPin2, ledState ? HIGH : LOW); delay(500);

digitalWrite(ledPin3, ledState ? HIGH : LOW); delay(500);

}

lastButtonState = currentButtonState;

}

## Output:

Conclusion:

Thus, learnt about basic components of IoT like Arduino UNO (blinking Arduino onboard LED), Breadboard , LED’s and interfacing Fade LED With Arduino Analog Output with connections of resistors, breadboard, Jumper wires & Arduino.

# Practical No. 22

AIM: To interface 16×2 LCD, Push Button, Potentiometer with Arduino and WAP to display message on LCD when push button is pressed.

## Objectives:

- To learn Arduino UNO basics

- To Learn Breadboard basics

- To Learn about 16x2 LCD.

- To Learn about Potentiometer

- WAP to display message on LCD when push button is pressed.

Components: Arduino UNO R3,Breadboard,1 16x2 LCD,4 Resistor,1 Potentiometer, wires.

## Circuit:

Program Code:

// include the library code:

#include <LiquidCrystal.h>

// initialize the library with the numbers of the interface pins LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

void setup()

{

pinMode(A0, INPUT); pinMode(A1, INPUT); pinMode(A2, INPUT);

}

void loop()

{

// set the cursor to column 0, line 1

//(note: line 1 is the second row, since counting begins with 0):

lcd.setCursor(0, 1);

if (digitalRead(A0) == HIGH)

{

lcd.print( "VIT MCA");

}

if (digitalRead(A1) == HIGH)

{

lcd.print("Sub-IoT");

}

if (digitalRead(A2) == HIGH)

{

lcd.print("Practicals");

}

}

## Output:

Conclusion:

Thus, learnt about basic components of IoT like Arduino UNO (blink Arduino onboard LED), Breadboard , 7 segment display and interfacing 16×2 LCD, Push Button, Potentiometer with Arduino and displaying message on LCD when push button pressed with connections of resistors, breadboard, Jumper wires & Arduino.

# Practical No. 23

Part 1 AIM: To upload data on Thingspeak cloud Manually.Steps:

- Go to Google and search for Thingspeak.

- If you are new to Thingspeak, Do sign up and make sure you are on your Channel page.

- Click on the NEW CHANNEL button (Green color) and create a new channel.

- Enter a channel name, any description of your choice, and make sure one field is selected or ticked and give that field a name of your choice. Click on save.

- Now in the private view, make sure you see a graph (empty).

- Now click on API KEYS tab, scroll down to find API requests section and in that copy the link of Write a Channel Feed and paste it in the Address bar of your browser. And press enter to get a blank screen with a number which indicates the number of data uploaded manually. Following is the example link: https://api.thingspeak.com/update?api_key=7JWB3X9H4O4VQGL7&field1=0

- Suppose you want to change the data to be entered in the graph, just change the

8. =0 to any value of your choice in the link. Above is the link, where we had changed 0 to 40.

https://api.thingspeak.com/update?api_key=7JWB3X9H4O4VQGL7&field1=40

9.  As a result, to see the visualization (graph), go to the private view and see the graph.

Part 2 AIM: To update readings to Thingspeak from Arduino using Tinkercad.Steps:

- Click on the NEW CHANNEL button (Green colour) and create a new channel.

- Enter a channel name, any description of your choice, and make sure one field is selected or tickedand give that field a name of your choice. Click on save.

- Now in the API Keys tab copy the Write API Key and Paste it in your program

## Circuit:

Program Code:

void setup() { Serial.begin(115200); delay(1000);

Serial.println("AT+CWJAP=\"Simulator Wifi\",\"\"");

delay(5000); // wait for connection

}

void loop() {

Serial.println("AT+CIPSTART=\"TCP\",\"a pi.thingspeak.com\",80");

delay(3000);

String cmd = "GET

/update?api_key=7JWB3X9H4O4VQGL7& field1=99 HTTP/1.1\r\n";

cmd += "Host: api.thingspeak.com\r\n"; cmd += "Connection: close\r\n\r\n";

Serial.print("AT+CIPSEND="); Serial.println(cmd.length()); delay(2000);

Serial.print(cmd);

delay(5000); Serial.println("AT+CIPCLOSE"); delay(20000); // wait before sending again

}

## Output:

NOTE:

- TEXT IN RED COLOUR IS YOUR WRITE API KEY

- Make sure you see your Serial Monitor and check if Data is sent to Thingspeak.

- To check result about data upload, go to thingspeak, click on private view and see the graph.

## Part 3

Aim: To interface Temperature sensor and ESP8266 with Arduino and update temperature reading toThingspeak.

## Steps:

- Click on the NEW CHANNEL button (Green colour) and create a new channel.

- Enter a channel name, any description of your choice, and make sure one field is selected or tickedand give that field a name of your choice. Click on save.

- Now in the API Keys tab copy the Write API Key and Paste it in your program

## Circuit:

PROGRAM Code:

void setup() { Serial.begin(115200); delay(1000);

// Connect to Wi-Fi Serial.println("AT+CWJAP=\"Simulator

Wifi\",\"\"\r\n");

delay(3000); // Wait for connection

// Convert voltage to Celsius (assuming TMP36 sensor)

float tempC = (volt - 0.5) * 100; // Adjust based on your sensor's characteristics

// Print the temperature to the Serial Monitor

Serial.println(tempC);

}

void loop() {

// Read the analog value from the sensor int sensorValue = analogRead(A0);

// Convert the analog value to voltage float volt = (sensorValue / 1024.0) * 5.0;

// Use 1024 for 10-bit resolution and 5V reference

// Start TCP connection to ThingSpeak

Serial.println("AT+CIPSTART=\"TCP\",\"a pi.thingspeak.com\",80\r\n");

delay(5000); // Wait for connection to establish

// Prepare the HTTP GET request String cmd = "GET

/update?api_key=63Z3U807XQRKWFWX

&field1=" + String(tempC) + " HTTP/1.1\r\n";

cmd += "Host: api.thingspeak.com\r\n"; cmd += "Connection: close\r\n\r\n";

// Send the length of the request

int len = cmd.length(); // Calculate the length of the command

Serial.print("AT+CIPSEND="); Serial.println(len);

delay(10); // Wait for the command to be processed

// Send the actual command Serial.print(cmd);

delay(100); // Wait for the request to be sent

// Close the TCP connection Serial.println("AT+CIPCLOSE\r\n"); delay(60000); // Wait before sending

again (60 seconds)

}

## Output:-

NOTE:

- TEXT IN RED COLOUR IS YOUR WRITE API KEY

- Make sure you see your Serial Monitor and check if Data is sent to Thingspeak.

- To check result about data upload, go to thingspeak, click on private view and see the graph.

# Practical No. 24

Aim: To interface LDR sensor, LED and ESP8266 with Arduino and update light intensity values to Thingspeak and tweet “LIGHT ON” message on tweeter when light intensity value is less than 300.

## Steps:

- Click on the NEW CHANNEL button (Green colour) and create a new channel.

- Enter a channel name, any description of your choice, and make sure one field is selected or tickedand give that field a name of your choice. Click on save.

- Now in the API Keys tab copy the Write API Key and Paste it in your program

## Circuit:-

PROGRAM:

int ldr=A0;//Set A0(Analog Input) for LDR. int value=0;

void setup()

{

Serial.begin(115200); pinMode(13,OUTPUT); delay(1000);

Serial.println("AT+CWJAP=\"Simulator Wifi\",\"\"\r\n");delay(3000);

}

void loop()

{

{

value=analogRead(ldr); Serial.println("LDR value is :"); Serial.println(value); if(value<300)

{

digitalWrite(13,HIGH);

}

else

{

digitalWrite(13,LOW);//Turns the LED OFF in Light.

}

Serial.println("AT+CIPSTART=\"TCP\",\"th ingspeak.com\",80");delay(5000);

int len = 65; Serial.print("AT+CIPSEN D=");Serial.println(len); delay(10);

Serial.print("GET/update?api_key=6WEDQNF N3GBKNCQ3&field1="+String(value)

+"HTTP/1.1\r\n");

delay(100); Serial.println("AT+CIPCLOSE=0\r\n");dela

y(6000);	}

}

## NOTE:

- TEXT IN RED COLOUR IS YOUR WRITE API KEY

- Make sure you see your Serial Monitor and check if Data is sent to Thingspeak.

- To check result about data upload, go to thingspeak, click on private view and see the graph.

- Once you finish doing the above steps go back to Thingspeak and next to the CHANNELS tab

,click on the APPS tab and select React option.

- Click on the NEW REACT button (Green colour) and give a React name. Here it is LDR React and do following settings:

- In the Action tab in the above figure, select ThingTweet option and then it will ask to link with your twitter account

- Link and Choose your Twitter Account.

- Save the React.

- Make sure your intensity of LDR is less than 300 in the TinkerCad LDR slider and then check your Twitter account for the Tweet of “LIGHT ON” message.

# Practical No. 25

Aim: To interface servo motor / DC motor with Arduino and WAP to sweep a servo back and forth through its full range of motion/to control a DC motor.

Components: Arduino UNO R3,Breadboard,4 Resistor,3 LED, 1 Push Button, Wires

## Circuit:

Code:

const int poten = A3; int var;

void setup()

{

Serial.begin(9600); pinMode(6, OUTPUT);

}

void loop()

{

var = analogRead(poten); analogWrite(6, var); Serial.println(var);

}

Output: